AI FINGERPRINT ATTENDANCE SYSTEM

MASTER PRODUCT REQUIREMENTS DOCUMENT

Implementation blueprint for Codex: firmware + cloud + WhatsApp + website + dashboard + operations

How to use this document Give this document to Codex together with the actual project repository/files. Codex should inspect first, then implement in phases, test after every phase, and never invent credentials or change the locked hardware wiring.

| Field | Value |
| --- | --- |
| Document status | Implementation-ready PRD / engineering instruction set |
| Version | 1.0 |
| Date | 19 September 2026 |
| Primary implementation agent | OpenAI Codex |
| Target hardware | ESP8266 NodeMCU ESP-12E + fingerprint sensor + I2C LCD + buzzer |
| Target cloud surface | Google Sheets/App Script initially; production backend + cloud database + web dashboard |
| Messaging | Meta WhatsApp Business / WhatsApp Cloud API, backend-only credentials |
| Primary users | Administrator, attendance operator/teacher, report viewer |

# Contents

1. Executive summary

2. Product vision and goals

3. Non-goals and hard constraints

4. Current system: what already works

5. Locked hardware and firmware contract

6. Current Google Sheets backend

7. Target architecture

8. Functional requirements

9. Data model

10. APIs and integration contracts

11. Registration flow

12. Attendance flow

13. Absence and cutoff logic

14. WhatsApp / Meta business messaging plan

15. Cloud data storage strategy

16. Website and admin dashboard

17. Reports and analytics

18. Security and secrets

19. Reliability, offline mode, retries and idempotency

20. Testing and acceptance criteria

21. Deployment and DevOps

22. Codex execution plan

23. Definition of done

24. Future-ready extensions

Appendix A. Master Codex prompt

Appendix B. Suggested project structure

Appendix C. Configuration checklist

Appendix D. Demo / evaluator checklist

# 1. Executive Summary

Build a complete biometric attendance platform around the existing ESP8266 fingerprint device. The device must remain stable and solder-compatible, while the software stack evolves from the currently working Google Apps Script + Google Sheets pipeline into a cloud-backed attendance system with a secure backend, Meta WhatsApp messaging, and a web-based admin dashboard.

The most important engineering principle is separation of concerns: the ESP8266 is an edge device for fingerprint capture and immediate user feedback; the backend is the source of truth for student/attendance data and messaging; the website is an administrative interface; Google Sheets is retained as a human-friendly operational/reporting surface and synchronization target during the transition.

Current state in one sentence Fingerprint enrollment and attendance scanning work; ESP-to-Google-Sheets logging is working (HTTP 302 from the web app and rows appearing in the sheet), but registration/master-data synchronization, mobile propagation, cloud persistence, messaging, and the website are not yet completed.

# 2. Product Vision and Goals

## 2.1 Vision

A reliable attendance platform where a student enrolls once, taps a fingerprint to record attendance once per day, the record is stored safely in the cloud, the official parent WhatsApp channel can send an attendance notification, and administrators can see current and historical data from a responsive website.

## 2.2 Primary goals

Make student registration a master-data operation, not an attendance event.

Make first successful attendance scan of the day the canonical attendance timestamp; repeated scans the same day must not create duplicate attendance entries.

Persist students, attendance events, device health, and notification outcomes in cloud storage.

Keep the working Google Sheets view synchronized for human access, exports, and demo friendliness.

Send attendance messages through the college-approved Meta WhatsApp Business setup using a backend, never by putting Facebook/Meta credentials on the ESP8266.

Provide a polished website dashboard for student management, attendance, reports, devices, settings, and message history.

Make the whole system observable, testable, idempotent, and recoverable when Wi-Fi, Google Apps Script, the cloud database, or Meta messaging temporarily fails.

## 2.3 Success outcomes

| Outcome | Expected behavior |
| --- | --- |
| New student registration | Creates local enrollment + cloud Students record; does not mark attendance. |
| Attendance scan | Finds student, stores first scan for the calendar day, shows confirmation on LCD, optionally triggers WhatsApp. |
| Duplicate scan | Returns already-present behavior without changing the first timestamp. |
| Absence | At configured cutoff, active students without attendance for the day become Absent. |
| Parent number | Stored once in student master record and propagated to attendance/notification logic. |
| Dashboard | Shows current day totals, present/absent, trends, student details, and system status. |
| Messaging | Backend sends an approved template and records message status/errors. |

# 3. Non-Goals and Hard Constraints

DO NOT break the physical device The ESP8266 wiring is soldered. Codex must not change pin mapping, sensor wiring, LCD I2C address, or hardware assumptions unless explicitly instructed later.

## 3.1 Locked hardware mapping

| Component | Locked configuration |
| --- | --- |
| Fingerprint sensor | SoftwareSerial: ESP D5 = RX, D6 = TX; sensor UART 57600 baud |
| I2C LCD 16x2 | Address 0x27; Wire.begin(D2, D1); D2 SDA, D1 SCL |
| Buzzer | D0 |
| Board | NodeMCU 1.0 (ESP-12E Module), ESP8266 Community boards 3.1.2 |
| Arduino IDE | 2.3.10 |
| Serial monitor | 115200 |

## 3.2 Hard software constraints

Do not introduce unrelated project code, URLs, names, databases, APIs, or dependencies.

Do not place Facebook/Meta passwords, permanent Meta access tokens, App Secrets, or other production secrets in firmware source code.

Keep Meta configuration as environment/config placeholders on the backend. In firmware, Meta can only be referenced as an integration comment/interface placeholder if needed.

Do not reintroduce any deprecated messaging provider code. Messaging must be abstracted behind a backend notification service.

Never print secrets to Serial Monitor, website logs, CI output, or error pages.

Prefer complete file replacements and coherent modules over fragile line-by-line edits.

Do not silently change existing sheet column semantics or date behavior without updating the entire integration contract.

# 4. Current System: What Already Works

Codex should treat these items as the baseline and preserve them unless an implementation change is explicitly required by this PRD.

| Area | Known current behavior | Codex instruction |
| --- | --- | --- |
| Fingerprint enrollment | Serial command R registers a student, scans the same finger twice, creates and stores a template, saves local database record. | Preserve UX and fix robustness, timeouts, and synchronization. |
| Local student DB | LittleFS file /students.txt; pipe-separated fields; max 30 students. | Keep backward compatibility while cloud master data is introduced. |
| Fingerprint ID | Auto-assigns a free ID and checks sensor templates to avoid orphan/template overwrite. | Do not regress ID allocation. |
| Attendance scan | Fingerprint match identifies a student and creates attendance payload. | Make cloud/backend event canonical and idempotent. |
| LCD | Rotating idle messages; scan/attendance feedback. | Keep the behavior and improve messages only where necessary. |
| Google Apps Script | Web app returns HTTP 302 and attendance records appear in the spreadsheet. | Do not throw away the working integration; evolve it safely. |
| Google Sheet | New workbook has Attendance, Students, Settings tabs. | Preserve them as operational surfaces. |
| Wi-Fi | ESP can connect to the configured network and obtains an IP. | Use bounded connection timeout and reconnect logic. |

# 5. Locked Hardware and Firmware Contract

## 5.1 Existing firmware responsibilities

Connect to Wi-Fi with a timeout; do not loop forever on connection failure.

Maintain NTP clock using Asia/Kolkata timezone or backend-confirmed time where feasible.

Provide commands: Attendance default, Registration (R), Clear (C + YES), Help (H).

Store enrollment metadata locally so attendance can continue in limited-connectivity scenarios.

Scan fingerprints and resolve fingerprint ID to student metadata.

Send attendance events to backend/cloud over HTTPS.

Avoid excessive Serial logging of routine fingerprint packet errors.

Be resilient to transient fingerprint UART error code 1 (FINGERPRINT_PACKETRECIEVEERR) without entering an infinite wait.

## 5.2 Local DB contract

| Field | Type | Meaning |
| --- | --- | --- |
| fingerprintID | uint8_t | Fingerprint template ID stored in sensor. |
| studentId | string | Institutional student identifier; unique. |
| studentName | string | Display name. |
| parentWhatsApp | string | Normalized parent WhatsApp number, including country code; do not assume local formatting is valid. |

Current local file format is: fingerprintID|studentId|studentName|parentWhatsApp. Codex may introduce a versioned format if necessary, but must support migration and never silently discard existing students.

# 6. Current Google Sheets Backend

## 6.1 Current workbook structure

| Sheet | Current purpose | Target role |
| --- | --- | --- |
| Attendance | One row per student plus a date column per day, with Present/Absent status. | Human-readable attendance register and reporting mirror. |
| Students | Master student database. | Canonical student list for admin operations / backup / audit. |
| Settings | Cutoff time and timezone. | Configuration mirror for non-code operators. |

## 6.2 Current Attendance columns

| Column | Meaning |
| --- | --- |
| A | Fingerprint ID |
| B | Student ID |
| C | Student Name |
| D | Mobile Number |
| E | Status |
| F onward | One date column per calendar day; time stored in the cell for attendance. |

## 6.3 Current Apps Script behavior

Validates a shared request key.

Opens a configured spreadsheet and creates/gets Attendance, Students, and Settings sheets.

Accepts fingerprint ID, student ID, student name, and optional parent WhatsApp/mobile number.

Creates a row for a new student and stores the mobile number when provided.

For existing students, updates fingerprint ID/name and only replaces mobile when a new value is explicitly supplied.

Creates a new date column when needed.

Prevents same-day duplicate attendance by keeping the first timestamp if that date cell is already populated.

Sorts attendance rows by Student ID.

Contains a daily absence trigger design around the configured cutoff time.

Known bug to fix immediately The ESP currently calls the Google logging function without passing parentWhatsApp/mobile data. The sheet therefore shows a blank Mobile Number even though the local record contains the number. Registration also currently stays local instead of synchronizing new students into the cloud Students master sheet.

# 7. Target Architecture

The final platform should separate device concerns, business logic, storage, messaging, and presentation. This makes it much easier to replace Google Sheets later without rewriting the fingerprint firmware.

ESP8266 biometric device ¦ HTTPS v Application Backend / API ¦--- Cloud Database (source of truth) ¦--- Google Sheets sync/export ¦--- Meta WhatsApp Cloud API ¦--- Scheduler / absence jobs v Web Admin Dashboard

## 7.1 Recommended responsibility split

| Layer | Owns | Must not own |
| --- | --- | --- |
| ESP8266 | Fingerprint hardware, local enrollment cache, device UX, scan event emission. | Meta access tokens, database credentials, website business rules. |
| Backend API | Authentication, validation, idempotency, business rules, cloud persistence, messaging orchestration. | Physical sensor handling. |
| Cloud DB | Students, attendance events, device status, notification logs, settings, audit logs. | UI logic. |
| Google Apps Script/Sheets | Human-readable spreadsheet mirror, export/report convenience, legacy compatibility. | Security-critical messaging secrets. |
| Meta WhatsApp | Outbound parent notification transport through approved business setup. | Student master database. |
| Website | Admin/operator UX, filters, reports, configuration, health status. | Directly storing secrets in browser code. |

# 8. Functional Requirements

| ID | Feature | Requirement |
| --- | --- | --- |
| FR-01 | Student registration | Operator can create a student with fingerprint ID, Student ID, name, and parent WhatsApp number. Registration must not write an attendance timestamp. |
| FR-02 | Duplicate student prevention | Student ID must be unique. Fingerprint ID must be unique among active enrolled templates. |
| FR-03 | Master sync | A successful registration must be reflected in local cache and cloud Students data; if cloud sync fails, the system must mark the sync state and retry rather than claim success. |
| FR-04 | Attendance scan | A recognized fingerprint creates an attendance event for the current local/authoritative calendar day. |
| FR-05 | Daily uniqueness | Only the first valid attendance event for a student on a date becomes the canonical first-scan timestamp. Later scans do not overwrite it. |
| FR-06 | Notification | A first successful attendance event may trigger a WhatsApp notification if messaging is enabled and recipient/template configuration is valid. |
| FR-07 | Notification logging | Every send attempt has a status: queued, sent/accepted, failed, retrying, or permanently failed, plus provider response metadata without storing sensitive tokens. |
| FR-08 | Absence | At cutoff, active students with no attendance for the date are marked Absent. |
| FR-09 | Dashboard | Admin can view today's totals and history, filter by student/date/status, and inspect individual student history. |
| FR-10 | Reports | Admin can export attendance by date range, student, or class/group as CSV and PDF where practical. |
| FR-11 | Device health | Dashboard shows online/offline/last-seen/last-event status for registered attendance devices. |
| FR-12 | Settings | Admin can configure timezone, daily cutoff, notification enablement, template identifiers, and device display settings, subject to role permissions. |
| FR-13 | Audit | Important mutations are auditable: enrollment, student update, manual attendance correction, absence run, notification configuration change. |
| FR-14 | Manual correction | Authorized admin can correct attendance with a reason; the original event should remain auditable rather than being silently deleted. |
| FR-15 | Offline resilience | If backend is unavailable, the ESP may cache unsent events locally and retry safely; duplicate retries must not create duplicate attendance. |

# 9. Data Model

## 9.1 Students

| Field | Required | Notes |
| --- | --- | --- |
| studentId | Yes | Unique institutional ID. |
| fingerprintId | Yes | Unique device template ID for active enrollment. |
| name | Yes | Student display name. |
| parentWhatsApp | Yes for notifications | Normalized international number; store separately from display name. |
| active | Yes | Controls absence calculations and device matching. |
| createdAt | Yes | Server timestamp. |
| updatedAt | Yes | Server timestamp. |
| lastSyncedToDevice | Optional | Useful for multi-device deployments. |

## 9.2 Attendance events

| Field | Meaning |
| --- | --- |
| eventId | Idempotency-safe unique event ID. |
| studentId | Student master reference. |
| fingerprintId | ID observed at scan. |
| deviceId | Unique ESP device identifier. |
| attendanceDate | Calendar date in configured timezone. |
| firstScanTime | Canonical first-scan time for that date. |
| status | Present / Absent / corrected states as needed. |
| receivedAt | Backend receipt time. |
| deviceTimestamp | ESP timestamp when available. |
| source | device / manual / import. |
| correctionReason | Required for manual corrections. |

## 9.3 Notifications

| Field | Meaning |
| --- | --- |
| notificationId | Unique record. |
| studentId | Recipient student. |
| parentWhatsApp | Masked/normalized recipient reference. |
| type | ATTENDANCE_PRESENT / other future types. |
| templateName/templateId | Approved Meta template reference. |
| variables | Safe template variables; do not log access tokens. |
| providerMessageId | Provider ID when returned. |
| status | queued / accepted / failed / retrying / delivered if available. |
| errorCode/errorMessage | Sanitized provider error. |
| createdAt/sentAt | Timestamps. |

## 9.4 Devices

| Field | Meaning |
| --- | --- |
| deviceId | Stable unique device identifier. |
| name | Friendly display name. |
| enabled | Whether device is allowed to send events. |
| lastSeenAt | Heartbeat/event timestamp. |
| lastFingerprintAt | Last successful scan time. |
| firmwareVersion | Reported firmware version. |
| ipAddress | Optional diagnostic value. |
| wifiRssi | Optional signal value. |

## 9.5 Audit logs

Record who/what/when for enrollment, edits, attendance corrections, configuration changes, absence jobs, and notification configuration. Do not store passwords, access tokens, or raw provider secrets in audit records.

# 10. APIs and Integration Contracts

## 10.1 Device registration API

POST /api/v1/devices/registration { "deviceId": "attendance-01", "fingerprintId": 4, "studentId": "STUDENT-004", "studentName": "Student Name", "parentWhatsApp": "+91..." } Response: 200/201 with canonical student record and sync status. Failure: structured error; ESP must not display success unless persistence is confirmed.

## 10.2 Attendance event API

POST /api/v1/attendance/events { "eventId": "attendance-01-4-2026-09-19T18:05:00", "deviceId": "attendance-01", "fingerprintId": 4, "studentId": "STUDENT-004", "attendanceDate": "2026-09-19", "deviceTimestamp": "2026-09-19T18:05:00+05:30" } Response should say one of: - recorded_first_scan - already_present - unknown_student - invalid_request - retryable_error - permanent_error

## 10.3 Device heartbeat API

POST /api/v1/devices/heartbeat { "deviceId": "attendance-01", "firmwareVersion": "1.x.x", "ipAddress": "10.x.x.x", "wifiRssi": -62 }

## 10.4 Website APIs

| Endpoint family | Examples |
| --- | --- |
| Auth | /auth/login, /auth/logout, /auth/me |
| Students | GET/POST/PATCH /api/v1/students |
| Attendance | GET /api/v1/attendance?from=...&to=...&student=... |
| Reports | GET /api/v1/reports/attendance.csv, POST /api/v1/reports/pdf |
| Notifications | GET /api/v1/notifications, POST /api/v1/notifications/test |
| Devices | GET /api/v1/devices, PATCH /api/v1/devices/:id |
| Settings | GET/PATCH /api/v1/settings |

# 11. Registration Flow

1. Operator sends the registration command or uses a future web registration workflow.

2. ESP validates Student ID, name, and parent number format locally.

3. ESP finds a free fingerprint template ID without overwriting existing sensor templates.

4. Student places finger for the first scan.

5. Fingerprint image is converted and the finger is removed.

6. Student places the same finger for the second scan.

7. ESP creates and stores the fingerprint model.

8. ESP writes the student to local LittleFS.

9. ESP sends the master student record to the backend/cloud Students store.

10. Backend upserts Students by Student ID and enforces uniqueness.

11. Backend mirrors/synchronizes the student to the Google Students sheet.

12. ESP receives a confirmed cloud result and displays registration success.

13. No Attendance row/date timestamp is created by this registration action.

Critical distinction Registration creates a student. Attendance records a presence event. These are separate transactions and must never be conflated.

# 12. Attendance Flow

1. Fingerprint sensor identifies a template ID.

2. ESP resolves the template to the local student record.

3. ESP builds an idempotent attendance event and sends it over HTTPS.

4. Backend validates device authorization and student existence.

5. Backend checks whether attendance already exists for that student and date.

6. If not present, backend creates canonical first-scan attendance and status Present.

7. Backend queues/dispatches a WhatsApp notification according to current settings.

8. Backend synchronizes the attendance result to Google Sheets.

9. ESP receives recorded_first_scan and shows success; or already_present and shows that attendance is already marked.

Idempotency rule Retrying the exact same event or scanning the same student again on the same date must never create two canonical attendance timestamps. The backend database must enforce this invariant, not merely the website.

# 13. Absence and Cutoff Logic

The system should treat absence as a scheduled derived state rather than an ESP scan event.

1. At the configured daily cutoff, the scheduler selects all active students.

2. For each student, it checks for any canonical attendance event for the date.

3. Students with attendance remain Present.

4. Students without attendance are marked Absent for the date.

5. The Google Attendance sheet receives the same status.

6. The dashboard records the absence job execution time and counts.

7. Re-running the absence job is idempotent and must not alter Present records.

Timezone must be configurable and default to Asia/Kolkata for this deployment.

Cutoff should be configurable (currently designed around 12:00).

Absence job must be safe across server restarts and scheduler duplication.

Manual attendance corrections after cutoff must be auditable.

# 14. WhatsApp / Meta Business Messaging Plan

## 14.1 Human-owned setup required

The Meta Business / WhatsApp account setup has administrative, identity, phone-number, and template-approval steps that a human organization administrator must perform. Codex can build and validate the integration around the resulting identifiers, but it should not invent or store organization credentials.

1. College/admin creates or confirms the Meta Business portfolio used for the attendance service.

2. College/admin creates or confirms the WhatsApp Business Account (WABA) and an appropriate business phone number.

3. College/admin completes any required business/phone verification and account setup steps shown in the Meta UI.

4. College/admin creates an approved outbound message template for attendance notifications, following current Meta template policies.

5. College/admin obtains the backend-required identifiers/configuration (for example WABA/phone number identifier, app/biz context, and appropriate access token or system-user credential).

6. Store these values only in backend environment variables / a secret manager. Never embed them in the ESP8266 firmware or frontend bundle.

7. Run a controlled test message from the backend to an authorized test recipient before enabling parent notifications in production.

## 14.2 Notification template contract

Use an approved template whose variables are deterministic and safe. Recommended semantic variables: student name, date, time, institution/device label. Avoid putting sensitive internal identifiers into parent-facing messages unless the college requires them.

Example semantic template payload { "template": "attendance_present", "language": "en", "variables": { "student_name": "{{studentName}}", "date": "{{date}}", "time": "{{time}}" } }

## 14.3 Backend messaging sequence

1. Attendance transaction commits successfully.

2. Backend creates notification job / log record.

3. Worker calls the current Meta WhatsApp Cloud API documented for the college account.

4. Backend stores provider response identifiers/status.

5. Transient failures are retried with exponential backoff and bounded attempts.

6. Permanent failures are surfaced on the dashboard for admin action.

7. The website exposes last notification attempt and status without exposing access tokens.

Current-API verification rule Before implementing the Meta request details, Codex must consult the then-current official Meta WhatsApp Cloud API documentation and use the college account’s actual current API version/requirements. Do not hard-code assumptions from old tutorials.

# 15. Cloud Data Storage Strategy

Google Sheets is already working and should remain available, but it should not be treated as the only durable database for the final platform. A real cloud datastore should become the source of truth once the backend is introduced.

## 15.1 Preferred target

Use a managed cloud database with server-side access control and indexed queries. A document database or SQL database are both acceptable; Codex should select the smallest production-appropriate option that fits the chosen backend stack and deployment environment.

## 15.2 Migration strategy

1. Keep the existing Apps Script + Sheets endpoint functioning.

2. Create the new backend API and database schema.

3. Import existing Students and Attendance data into the cloud database with a one-time migration tool.

4. Add dual-write / sync from the new backend into Google Sheets.

5. Run reconciliation checks comparing cloud totals to Sheets totals.

6. Switch the ESP to the new backend only after end-to-end tests pass.

7. Keep the old Apps Script path available temporarily as a rollback/fallback if practical.

## 15.3 Cloud storage requirements

Server-generated timestamps plus device timestamps when available.

Unique constraints / idempotency keys for attendance.

Indexes for student ID + date and attendance date range queries.

Backups or provider-managed retention sufficient for the project.

Soft-delete/archive instead of destructive deletes for students whenever possible.

Audit trail for edits and corrections.

# 16. Website and Admin Dashboard

## 16.1 Target users

| Role | Typical access |
| --- | --- |
| Admin | Full access: students, attendance corrections, settings, devices, reports, notifications, users. |
| Operator/Teacher | Attendance and student lookup; limited corrections; no credential/configuration management. |
| Viewer | Read-only dashboard and reports. |

## 16.2 Required pages

| Page | Required UI |
| --- | --- |
| Login | Secure authentication, session handling, sign out. |
| Dashboard | Today Present, Today Absent, total students, attendance rate, recent scans, device health, notification summary. |
| Students | Search, filter, add, edit, activate/deactivate, fingerprint ID, student ID, parent WhatsApp, enrollment date. |
| Student detail | Profile + attendance history + notification history + audit events. |
| Attendance | Date picker, range filter, search, Present/Absent filters, student filter, device filter, manual correction flow. |
| Reports | Daily/monthly/custom range reports, CSV export, printable/PDF-friendly view. |
| Notifications | Delivery/failed status, search, retry, test configuration, template metadata. |
| Devices | Online/offline, last seen, firmware, IP, RSSI, last scan, enable/disable. |
| Settings | Timezone, cutoff, notification toggle, template config, institution label, retention/reporting settings. |
| Audit log | Who changed what and when. |

## 16.3 Dashboard UX requirements

Responsive layout for laptop and mobile browser.

Clear hierarchy: today first, trends second, operational details third.

Use tables for exact records and charts for trends.

Always display date/time with timezone context.

Use explicit statuses rather than ambiguous colors alone; maintain accessible contrast.

Never expose raw secrets in the browser or HTML source.

Errors should be actionable: what failed, whether it is retryable, and what the operator should do.

# 17. Reports and Analytics

| Report | Required fields / behavior |
| --- | --- |
| Daily attendance | Student ID, name, Present/Absent, first-scan time, device, notification status. |
| Monthly attendance | Per-student day grid, total present, total absent, attendance percentage. |
| Student history | Chronological attendance and correction history. |
| Notification report | Recipient, template, sent time, provider ID, current status, failure reason. |
| Device report | Last-seen, scan count, connectivity events, firmware. |
| Export | CSV at minimum; PDF/print view where practical. |

## 17.1 Analytics cards

Total active students.

Present today.

Absent today.

Attendance rate today.

Attendance trend over last 7/30 days.

Notifications sent / failed today.

Online devices / total devices.

# 18. Security and Secrets

Credential rule Existing chat/code history contains credentials that must be treated as exposed. Rotate/revoke any real password, API secret, access token, or auth credential that has been pasted or screenshot-shared, then replace it with environment-managed secrets.

## 18.1 Never put these in firmware

Meta/Facebook account passwords.

Permanent Meta access tokens.

Meta App Secret.

Cloud database admin credentials.

Website admin passwords.

Provider signing secrets that are not required at device level.

## 18.2 Device authentication

Use per-device credentials or keys where practical. At minimum, the API must authenticate the device and validate request structure, timestamps, and replay/idempotency. Use HTTPS/TLS and keep all network requests bounded by timeouts.

## 18.3 Backend security

Environment variables or managed secret store for secrets.

Role-based access control for the website.

Rate limiting for public endpoints.

Input validation and output encoding.

No secrets in logs.

Sanitized provider errors.

Audit logs for privileged actions.

# 19. Reliability, Offline Mode, Retries and Idempotency

## 19.1 ESP network behavior

Wi-Fi connect timeout; show a clear offline state instead of endless dots.

Automatic reconnect with backoff.

Queue a small number of attendance/registration sync tasks in LittleFS if network is down.

Each queued event needs a stable event ID and retry count.

Successful replay must be idempotent at the backend.

## 19.2 Backend behavior

Database write is the source of truth before sending notification.

WhatsApp failure must not roll back a valid attendance record.

Google Sheets failure must not roll back a valid cloud attendance record.

Retry notification/sheet sync independently of attendance creation.

Expose reconciliation health metrics on the dashboard.

## 19.3 Failure matrix

| Failure | Expected behavior |
| --- | --- |
| Fingerprint sensor transient packet error | Retry/timeout; no infinite loop. |
| Wi-Fi unavailable | Device enters offline mode; scans may queue locally. |
| Backend 5xx/time-out | Keep event queued and retry. |
| Unknown student | Do not create attendance; show unregistered fingerprint error. |
| Duplicate same-day event | Return already_present; preserve first timestamp. |
| Google Sheets unavailable | Attendance remains in cloud; mark sync pending and retry. |
| Meta unavailable | Attendance remains valid; mark notification failed/retry. |
| Scheduler runs twice | Idempotent absence job; Present never becomes Absent. |

# 20. Testing and Acceptance Criteria

## 20.1 Firmware tests

Register student #1, #2, #3, #4 consecutively; every student persists and unique fingerprint IDs are assigned.

Restart device; all local student records remain readable.

Clear sensor/database only on explicit YES confirmation.

Attendance scan works after boot without manual recovery.

Transient fingerprint packet error does not cause infinite waiting.

Wi-Fi timeout does not hang the device indefinitely.

Repeated scan same day produces already-present behavior.

## 20.2 Backend/API tests

Valid registration creates/upserts student once.

Duplicate Student ID is rejected or handled by explicit idempotent update semantics.

Attendance uniqueness is enforced by database constraint / transaction.

Retries cannot create duplicate attendance.

Unknown device is rejected.

Unknown fingerprint/student is rejected safely.

Absence job is repeatable and idempotent.

Secrets are absent from logs and response bodies.

## 20.3 Web tests

Admin login/logout works.

Role permissions are enforced server-side.

Student CRUD updates cloud data and audit log.

Attendance filters return correct date range.

Reports match database totals.

Device status refreshes.

Notification history is accurate.

## 20.4 End-to-end acceptance scenario

1. Create a new student on the ESP.

2. Verify the Students cloud record exists and Google Students row appears.

3. Scan the finger before cutoff.

4. Verify exactly one attendance record for the date.

5. Verify Google Attendance shows the first-scan time.

6. Verify a WhatsApp send attempt is logged by the backend.

7. Scan the same finger again.

8. Verify timestamp remains unchanged and no second attendance event is created.

9. Create a second student and confirm independent attendance behavior.

10. Run absence job and confirm only students without attendance become Absent.

11. Open dashboard and verify totals, student history, and notification status.

# 21. Deployment and DevOps

## 21.1 Repositories / environments

| Environment | Purpose |
| --- | --- |
| local | Codex development and tests. |
| staging | Test with Meta test recipient, staging DB, test spreadsheet. |
| production | College live system; protected secrets and backups. |

## 21.2 Required environment variables (names only)

APP_ENV= PORT= DATABASE_URL= JWT_SECRET= DEVICE_API_KEY_OR_SECRET= GOOGLE_SHEETS_WEB_APP_URL= GOOGLE_SHEETS_SHARED_KEY= GOOGLE_SERVICE_ACCOUNT_* (if used) META_WABA_ID= META_PHONE_NUMBER_ID= META_ACCESS_TOKEN= META_API_VERSION= META_TEMPLATE_NAME= META_TEMPLATE_LANGUAGE= TIMEZONE=Asia/Kolkata ATTENDANCE_CUTOFF=12:00

Use placeholder values in source-control examples. Actual values belong in .env locally (gitignored), deployment secret stores, or environment configuration.

## 21.3 CI/CD

Run formatter/linter/type checks.

Run unit/integration tests.

Run database migration checks.

Build frontend.

Build backend.

Validate firmware compilation with the locked board settings.

Block deployment when tests fail.

# 22. Codex Execution Plan

Codex is capable of end-to-end code changes, refactors, tests, and web development, so this project should be handed to it as a staged engineering program rather than one blind mega-edit. The official Codex guidance describes it as an agent for complex refactors and end-to-end engineering work; use a repository with the real files, not only screenshots or chat snippets.

## Phase 0 - Protect the baseline

Create a Git commit/tag or zipped backup of the last known-working firmware and Apps Script.

Inventory every file and identify which file is compiled/uploaded by Arduino IDE.

Do not edit until architecture and dependencies are mapped.

## Phase 1 - Fix the current firmware + Apps Script system

Pass parentWhatsApp from ESP to the Google endpoint.

Synchronize successful registrations into Students sheet.

Ensure registrations #2/#3/#4 create separate master rows.

Ensure registration never marks attendance.

Preserve first-scan-per-day timestamp and duplicate protection.

Fix timeouts/retries and eliminate infinite loops.

Add structured backend response handling.

## Phase 2 - Build production backend + cloud database

Define schema/migrations.

Implement authenticated device API.

Implement student and attendance services.

Implement idempotency.

Implement Google Sheets sync.

Implement scheduler for absence.

Implement notification service interface.

## Phase 3 - Meta WhatsApp integration

Verify current Meta Cloud API documentation before coding the request format.

Add environment-driven Meta client.

Add approved template sending.

Add retry/status logging.

Add a safe admin test-send endpoint.

Add notification history UI.

## Phase 4 - Website/dashboard

Implement auth and RBAC.

Implement dashboard.

Implement students CRUD.

Implement attendance views.

Implement reports.

Implement devices.

Implement notifications.

Implement settings/audit.

## Phase 5 - Hardening and handover

End-to-end test on actual hardware.

Reconcile database vs Google Sheets.

Test offline/retry scenarios.

Rotate all development credentials before production.

Document deployment and recovery procedures.

Produce a final demo checklist.

# 23. Definition of Done

Four consecutive student registrations succeed and appear in cloud Students + Google Students without any attendance timestamps.

Attendance scan creates exactly one canonical event per student per date.

Repeated scan the same day never overwrites the first attendance time.

Parent WhatsApp data is stored once in Students and available to notification logic.

Meta messaging is implemented only in the secure backend and can be enabled/disabled from configuration.

Absence job reliably marks missing attendance after cutoff.

Dashboard provides students, attendance, reports, devices, notifications, settings, and audit visibility.

Cloud database is the durable source of truth; Google Sheets is synchronized/operational.

All secrets are externalized and no production credential is committed to source control.

Firmware retains the locked hardware mapping and remains compilable/uploadable with the current board settings.

All critical flows have automated tests plus one real-device end-to-end test.

Recovery instructions exist for backend outage, sheet outage, Meta outage, and device offline mode.

# 24. Future-Ready Extensions

Multiple ESP attendance devices with one cloud student master.

Class/department/semester/group fields.

Late / half-day / leave status rules.

Student self-service profile view.

Parent notification preferences.

Multiple WhatsApp templates for absent/late/leave notices.

Monthly automated reports.

QR fallback or secondary identification method.

Device remote configuration and firmware update strategy.

Data retention and archival policies.

Institution branding and multi-campus tenancy if the college expands the system.

# Appendix A. Master Codex Prompt

Copy the following prompt into Codex after attaching/providing the real repository/project files. It is intentionally strict so Codex audits before editing.

You are the lead engineer responsible for completing the AI Fingerprint Attendance System described in the attached Master PRD. MISSION Build the system end-to-end without breaking the currently working fingerprint attendance device. Work from the actual repository/files. Do not rely on screenshots when the source code can be inspected. NON-NEGOTIABLE CONSTRAINTS 1. Preserve the locked hardware wiring: - Fingerprint: ESP D5 RX, D6 TX, SoftwareSerial, sensor 57600 - LCD: 16x2 I2C address 0x27, Wire.begin(D2, D1) - Buzzer: D0 - Board: NodeMCU 1.0 ESP-12E 2. Do not rewire hardware. 3. Do not introduce unrelated project code, URLs, APIs, databases, or names. 4. Do not put Meta/Facebook passwords, permanent Meta access tokens, database admin credentials, or other production secrets in ESP firmware or frontend bundles. 5. Use placeholders in examples and environment variables/secret stores for real credentials. 6. Do not expose secrets in logs. 7. Registration and attendance are separate transactions. 8. First attendance scan per student per calendar day is canonical. Duplicate scans must never overwrite the first timestamp. 9. Maintain backward compatibility with the current LittleFS /students.txt format or provide safe migration. 10. Preserve the working Google Apps Script + Google Sheets integration while implementing the new cloud/backend architecture. CURRENT BASELINE TO PRESERVE - Fingerprint enrollment via serial command R, including two scans and storeModel. - Clear via C + YES. - Help via H. - LittleFS local student DB, MAX_STUDENTS 30. - Wi-Fi and NTP with bounded timeouts; never infinite wait loops. - LCD idle rotation and scan feedback. - Google Apps Script currently returns HTTP 302 and the Attendance sheet receives rows. - Google workbook contains Attendance, Students, and Settings sheets. PHASE 1: FIX CURRENT CODE FIRST A. Pass parentWhatsApp/mobile from ESP to Apps Script so Mobile Number is populated. B. On successful registration, sync the new student to the cloud/Students master list. C. Registration #2/#3/#4 must create independent student records; never overwrite earlier students. D. Registration must never create an attendance timestamp. E. Duplicate attendance same day must keep the first timestamp. F. Make Wi-Fi, fingerprint and HTTP operations bounded, retryable, and non-blocking where practical. G. Add structured response handling so the ESP distinguishes recorded_first_scan, already_present, unknown_student, retryable_error, and permanent_error. H. Add tests for four registrations, duplicate attendance, reboot persistence, and transient sensor/network failures. PHASE 2: BACKEND + CLOUD DATABASE Choose a maintainable production-appropriate backend stack based on the repository and deployment environment. Prefer a boring, well-supported stack over unnecessary complexity. Implement: - authenticated device API - student CRUD/master data - attendance event API - idempotency / unique date constraint - device heartbeat - scheduled absence job - notification service abstraction - Google Sheets synchronization - audit log - migrations/schema - environment-driven configuration DATA REQUIREMENTS Students: studentId, fingerprintId, name, parentWhatsApp, active, createdAt, updatedAt. Attendance: eventId, studentId, fingerprintId, deviceId, attendanceDate, firstScanTime, status, receivedAt, deviceTimestamp, source, correctionReason. Notifications: notificationId, studentId, parentWhatsApp, type, templateName/templateId, variables, providerMessageId, status, errorCode/errorMessage, createdAt, sentAt. Devices: deviceId, name, enabled, lastSeenAt, lastFingerprintAt, firmwareVersion, ipAddress, wifiRssi. Audit: actor, action, target, timestamp, before/after metadata where appropriate. PHASE 3: META WHATSAPP BUSINESS INTEGRATION Before writing the provider request code, verify the current official Meta WhatsApp Cloud API documentation and the current API requirements for the college account. Do NOT ask for or store a Facebook password. Expect the human administrator to provide/configure the required business identifiers, WhatsApp phone number identifier, approved message template information, language, API version, and secret/token through secure environment configuration. Implement: - Meta client service - approved template sending - timeout/retry/backoff - provider response parsing - notification log - safe test-send endpoint - enable/disable setting Never make attendance depend on WhatsApp success. Attendance is valid even if WhatsApp fails. PHASE 4: WEBSITE + DASHBOARD Build a responsive admin/operator dashboard with: - login/logout - RBAC: Admin, Operator/Teacher, Viewer - dashboard overview - student list/search/filter/create/edit/activate/deactivate - student detail + history - attendance daily and date-range view - manual correction with required reason and audit - reports + CSV/PDF-friendly output - notification history and retry - device health/status - settings: timezone, cutoff, notification enablement, template configuration - audit log UI PRINCIPLES Keep the UI clean and professional. Prioritize today's attendance, clear table data, search/filter, and operational status. Do not put secrets in client-side code. PHASE 5: RELIABILITY Implement safe offline/retry behavior on the ESP where feasible. Every queued event needs a stable ID. Backend must be idempotent. Google Sheets and Meta are downstream systems; their failures must not roll back the canonical attendance record. TESTING Run: - firmware compile with locked Arduino/ESP8266 configuration - backend unit/integration tests - database migration checks - API contract tests - frontend build/type/lint tests - end-to-end tests for registration -> Students -> attendance -> notification -> dashboard - failure tests for duplicate, offline, provider error, sheet error, scheduler rerun CODING RULES - Inspect first. Plan second. Edit third. - Prefer small, reviewable commits/patches. - After each phase, run tests and report exactly what passed/failed. - Do not silently change hardware pins or data formats. - Do not delete working code just because a rewrite is easier. - Preserve existing behavior where it is not in conflict with the PRD. FINAL OUTPUT REQUIRED 1. List every file changed/created. 2. Explain the final architecture. 3. Provide exact run/build/test commands. 4. Provide environment-variable template with placeholders only. 5. Provide database migration/setup steps. 6. Provide Meta configuration steps that must be done by a human admin. 7. Provide firmware upload settings. 8. Provide an end-to-end verification checklist. 9. State any remaining manual steps or blockers. 10. Do not claim success for a component that was not actually tested.

# Appendix B. Suggested Project Structure

attendance-system/ ├─ firmware/ │ ├─ attendance_device.ino │ ├─ include/ │ └─ README.md ├─ backend/ │ ├─ src/ │ │ ├─ api/ │ │ ├─ services/ │ │ ├─ db/ │ │ ├─ jobs/ │ │ ├─ providers/ │ │ └─ utils/ │ ├─ migrations/ │ ├─ tests/ │ └─ .env.example ├─ web/ │ ├─ src/ │ ├─ public/ │ └─ tests/ ├─ integrations/ │ └─ google-apps-script/ │ └─ Code.gs ├─ scripts/ │ ├─ migrate-sheet-data.* │ └─ reconcile-data.* ├─ docs/ │ ├─ API.md │ ├─ DEPLOYMENT.md │ └─ META_SETUP.md ├─ .gitignore ├─ README.md └─ PRD_AI_Attendance_Codex_Master.md

# Appendix C. Configuration Checklist

| Item | Owner | Status at start | Notes |
| --- | --- | --- | --- |
| Fingerprint wiring | Hardware | LOCKED | Do not change. |
| ESP firmware baseline | Developer/Codex | WORKING | Back up before edits. |
| Google Apps Script | Developer/Codex | WORKING | Preserve during migration. |
| Google Sheets workbook | Admin/Developer | READY | Attendance + Students + Settings. |
| Cloud database | Developer/Codex | TO BUILD | Select managed datastore. |
| Backend hosting | Developer/Codex | TO BUILD | Choose deploy target. |
| Meta Business/WABA | College admin | TO CONFIGURE | Human-owned setup. |
| WhatsApp sender | College admin | TO CONFIGURE | Business phone number required. |
| Approved template | College admin | TO CONFIGURE | Attendance message template. |
| Meta secrets | College admin/Developer | TO CONFIGURE | Backend secret store only. |
| Website domain | Admin/Developer | TO CONFIGURE | Optional for first demo, required for public deployment. |
| Admin accounts | Admin | TO CONFIGURE | Use RBAC. |

# Appendix D. Demo / Evaluator Checklist

1. Power on ESP8266 and show boot + Wi-Fi state.

2. Register a new student and show that Students master record appears without attendance.

3. Scan the same student and show Present + first timestamp.

4. Scan again and show Already Present while timestamp remains unchanged.

5. Scan a second student and show separate record.

6. Open dashboard and show today's counts and recent scan.

7. Open student detail and show attendance history.

8. Show notification log / successful parent-message workflow if Meta has been enabled.

9. Show Google Sheets mirror for Attendance and Students.

10. Demonstrate cutoff/absence logic on test data.

11. Show device health and last-seen state.

# Final Engineering Note

This PRD is intentionally broader than a firmware fix. The intended end state is a maintainable attendance product: biometric edge device -> secure backend -> cloud database -> Google Sheets operational mirror + Meta WhatsApp notification service + responsive admin dashboard. Codex should execute the work in stages, preserving the working baseline and proving each stage with tests before moving forward.

Build it like a real system, not a collection of patches.